Breakout Board - Ar duino & MicroPython Compatible pin layout for esp32 super mini c3

For the common ESP32-C3 Super Mini, a breakout board simply brings the castellated GPIO/power pads to breadboard-friendly headers. Arduino and MicroPython use the same physical GPIO numbers—only the code syntax differs. The board has 16 pins, with 11 general GPIOs plus 3V3, 5V, and GND.

Typical pin layout

Orient the board with the USB-C connector at the bottom. Clone boards can vary slightly, so confirm against the silkscreen on the underside of your specific board.

| Left side, bottom → top | Right side, bottom → top |
| --- | --- |
| 5V | GND |
| GND | 3V3 |
| GPIO 0 / ADC | GPIO 10 |
| GPIO 1 / ADC | GPIO 9 / BOOT / strapping |
| GPIO 2 / ADC / strapping | GPIO 8 / onboard LED / strapping |
| GPIO 3 / ADC | GPIO 7 / SPI CS |
| GPIO 4 / SPI SCK | GPIO 6 / SPI MOSI |
| GPIO 5 / SPI MISO | GPIO 20 / UART RX |
| — | GPIO 21 / UART TX |

Some vendors show 8 pins per side rather than this mirrored reference layout, but the important point is: wire and program by the GPIO number printed on the board, not by Arduino-style “D” labels.

Recommended assignments

| Function | GPIO | Notes |
| --- | --- | --- |
| I²C SDA | 8 | Default convention; shares onboard active-low LED |
| I²C SCL | 9 | BOOT/strapping pin—use with care |
| SPI SCK | 4 | Standard C3 Super Mini convention |
| SPI MISO | 5 |  |
| SPI MOSI | 6 |  |
| SPI CS | 7 | Choose another safe GPIO if needed |
| UART RX | 20 | Default UART receive |
| UART TX | 21 | Default UART transmit |
| Analog inputs | 0, 1, 2, 3 | Use 0/1 first; GPIO2 affects boot strapping |
| Onboard LED | 8 | Usually active low: LOW turns it on |

These mappings are commonly used for the Super Mini, including SPI 4/5/6/7, UART 20/21, and LED GPIO8.

Pins to avoid for external devices

Avoid—or use only when you understand the boot requirements:

- GPIO2: boot/strapping pin; an attached circuit that holds it at the wrong level can prevent normal boot.

- GPIO8: boot/strapping pin and onboard LED; external loads may interfere with boot or make LED behavior confusing.

- GPIO9: BOOT button/strapping pin; avoid connecting a device that can pull it low during reset.

- GPIO20/21: safe for general use, but reserve them if you want a dedicated UART connection.

GPIO2, GPIO8, and GPIO9 are identified as strapping-related pins on this board family.

Arduino examples

Blink the onboard LED

cpp

#define LED_PIN 8

void setup() {

pinMode(LED_PIN, OUTPUT);

}

void loop() {

digitalWrite(LED_PIN, LOW);   // LED on: active-low

delay(500);

digitalWrite(LED_PIN, HIGH);  // LED off

delay(500);

}

I²C sensor wiring

- Sensor VCC → 3V3

- Sensor GND → GND

- Sensor SDA → GPIO8

- Sensor SCL → GPIO9

cpp

#include <Wire.h>

void setup() {

Wire.begin(8, 9);  // SDA, SCL

}

void loop() {

}

In Arduino IDE, choose ESP32C3 Dev Module. If serial output/upload behavior is unreliable, enable Tools → USB CDC On Boot → Enabled.

MicroPython equivalents

Blink

python

from machine import Pin

from time import sleep

led = Pin(8, Pin.OUT)

while True:

led.off()       # active-low LED: off() drives LOW, LED turns on

sleep(0.5)

led.on()        # drives HIGH, LED turns off

sleep(0.5)

I²C

python

from machine import I2C, Pin

i2c = I2C(0, scl=Pin(9), sda=Pin(8), freq=400_000)

print(i2c.scan())

SPI

python

from machine import SPI, Pin

spi = SPI(

1,

baudrate=1_000_000,

polarity=0,

phase=0,

sck=Pin(4),

mosi=Pin(6),

miso=Pin(5)

)

cs = Pin(7, Pin.OUT, value=1)

Electrical cautions

- ESP32-C3 GPIO is 3.3 V logic only. Do not feed a GPIO directly with 5 V.

- Power the board through USB-C, the 5V pin, or a regulated 3V3 pin—never connect both 5 V and 3.3 V external supplies indiscriminately.

- Use a level shifter for 5 V sensors, UART devices, or SPI modules.

- Keep total current demand modest; the 3V3 regulator is intended for lightweight sensors, not motors, relays, or high-current LED strips.

If you mean a specific commercial breakout board, share a photo or product link—the physical header order can differ among ESP32-C3 Super Mini clones.

where can i find a picture of the pin layout of breadboard

The most useful picture is a labeled ESP32-C3 Super Mini pinout diagram, which shows every physical header pin and its GPIO function. This is what you use to place the board on a breadboard and connect jumper wires correctly.

You can open the source page here: Random Nerd Tutorials—ESP32-C3 Super Mini pinout. It includes the pin layout and Arduino setup information.

For a real breadboard wiring example, this image shows the Super Mini connected with jumper wires to another module:

Breadboard placement

Place the ESP32-C3 Super Mini across the center trench of a standard solderless breadboard:

text

Breadboard rows

A B C D E   gap   F G H I J

┌───────────┐

│ ESP32-C3  │

│ Super Mini│

└───────────┘

Each side’s pins must land on separate rows; otherwise pins on opposite sides can be electrically shorted together. Use male header pins soldered to the board, then connect sensors with male-to-male Dupont jumpers.

For wiring reference:

- Connect sensor VCC → 3V3

- Connect sensor GND → GND

- Use GPIO labels—not physical row numbers—for signal connections

- Do not connect 5 V directly to a GPIO pin; ESP32-C3 signals are 3.3 V logic.